Exponentiation by Squaring (Java / Python)

Implement expo(a, b, c) that computes a^b mod c.

Constraints: 1 <= a <= 100, 0 <= b <= 10^9, 1 <= c <= 10^9.

The algorithm must run in O(log b) time using exponentiation by squaring:

  If b == 0: return 1 % c
  If b is odd:  a^b = a * a^(b-1)
  If b is even: a^b = (a^(b/2))^2

Apply modulo c at each step to keep intermediate values bounded.
The solution must not contain the strings "import" or "**".

Rubric

General marking note:

No double deduction (except in point 5). Do not deduct again for an error already penalised in an earlier point. For example: if the base case is wrong, base the overall correctness marks on whether the recursive steps are otherwise consistent.

This rubric applies to both Java and Python submissions. The Java method is expo(long a, long b, long c); the Python function is expo(a, b, c). Both are correct.

The use of 'import' and '**' (Python power operator) is not allowed. Penalise in point 5 if either appears in the submission.

The algorithm must run in O(log b) recursive depth. Iterative implementations using the same even/odd case split are also acceptable.

1. Base case: b == 0: /20

Target: When b == 0, the function returns 1 % c (handles termination and the edge case c == 1).

base case (Wrong): 0/20: No base case is present; the recursion never terminates or returns an incorrect constant (e.g., always returns 0 or 1 regardless of c).

base case (Semi): 10/20: A base case is present but incorrect: for example, returns 1 without applying % c (fails when c == 1), or the base condition is b == 1 instead of b == 0, causing incorrect results for b == 0.

base case (Correct): 20/20: Returns 1 % c when b == 0, correctly terminating the recursion and handling the edge case where c == 1.

2. Parity detection: odd vs even b: /15

Target: The algorithm correctly distinguishes odd b from even b using b % 2 (or equivalent).

parity (Wrong): 0/15: No parity check is present; odd and even b are treated identically, so only one recursive case is ever applied.

parity (Semi): 8/15: A parity check is present but the odd and even cases are swapped (e.g., b % 2 == 0 is treated as the odd case), producing incorrect results for all non-trivial inputs.

parity (Correct): 15/15: b % 2 != 0 (or b % 2 == 1) correctly identifies odd b; b % 2 == 0 correctly identifies even b. The two cases lead to different recursive branches.

3. Odd case: reduce by 1 and multiply: /20

Target: When b is odd, the function returns (a * expo(a, b-1, c)) % c.

odd case (Wrong): 0/20: The odd case is absent, or reduces b by a value other than 1, or does not multiply by a.

odd case (Semi): 10/20: Reduces b by 1 and recurses correctly, but does not multiply by a (e.g., returns expo(a, b-1, c) directly), or does not apply % c to the result.

odd case (Correct): 20/20: Returns (a * expo(a, b-1, c)) % c, correctly factoring out one a and reducing the exponent by 1.

4. Even case: halve and square: /25

Target: When b is even, the function computes tmp = expo(a, b/2, c) and returns (tmp * tmp) % c.

even case (Wrong): 0/25: The even case is absent, or does not halve b, or does not square the recursive result (e.g., just returns expo(a, b/2, c) without squaring).

even case (Semi): 12/25: Halves b and recurses, but squares incorrectly (e.g., multiplies by a instead of by itself, or calls expo twice with b/2 which is correct but doubles the recursive work to O(b)), or does not apply % c to the squared result.

even case (Correct): 25/25: Computes tmp = expo(a, b//2, c) once (using integer division), then returns (tmp * tmp) % c, achieving O(log b) depth and correctly squaring the half-power result.

5. Overall correctness: /20

Target: expo(a, b, c) returns the correct value of a^b mod c for all valid inputs, without using 'import' or '**'.

Overall (Wrong): 0/20: Returns incorrect results for most inputs, or contains 'import' or '**' in the submission.

Overall (Semi): 10/20: Returns correct results for small or simple inputs (e.g., b=0, b=1, small a and b) but fails for large b due to integer overflow, missing modulo steps, or incorrect recursion depth.

Overall (Correct): 20/20: Returns a^b mod c correctly for all valid inputs; 'import' and '**' are absent; modulo is applied at each recursive step to keep intermediate values bounded.

Overall (Internally consistent but incorrect final result): 15/20: Logic follows consistently from the student's own base case and recursive steps, but produces incorrect output due to a systematic error (e.g., missing modulo at one step, or b/2 using float division).

TOTAL MARK: SUM/100

Sample Solution -- Java

long expo(long a, long b, long c) {
    if (b == 0) {
        return 1 % c;
    } else if (b % 2 != 0) {
        return (a * expo(a, b - 1, c)) % c;
    } else {
        a = expo(a, b / 2, c) % c;
        return (a * a) % c;
    }
}

Explanation:

Point 1 (base case): b == 0 returns 1 % c. The % c handles the edge case where c == 1 (result must be 0).

Point 2 (parity): b % 2 != 0 detects odd b; the else branch handles even b. These two branches cover all b > 0.

Point 3 (odd case): (a * expo(a, b-1, c)) % c reduces b by 1 and multiplies by a, applying % c to keep the result bounded.

Point 4 (even case): expo(a, b/2, c) is called once, stored in a, then (a * a) % c squares the result. Calling expo once (not twice) keeps the recursion depth O(log b).

Mark for this sample: 100/100

Sample Solution -- Python

def expo(a, b, c):
    if b == 0:
        return 1 % c
    elif b % 2:
        return (a * expo(a, b - 1, c)) % c
    else:
        a = expo(a, b // 2, c) % c
        return (a * a) % c

Explanation:

Point 1 (base case): b == 0 returns 1 % c, terminating the recursion correctly for all values of c.

Point 2 (parity): b % 2 is truthy for odd b (Python shorthand for b % 2 != 0); else handles even b.

Point 3 (odd case): (a * expo(a, b-1, c)) % c. Uses b-1 to reduce to an even exponent, then multiplies by a.

Point 4 (even case): a = expo(a, b // 2, c) % c stores the half-power result; (a * a) % c squares it. Integer division b // 2 avoids floats.

Mark for this sample: 100/100